## Procesamiento de VSP. (Primeros arribos y espectros)

import segyio
import numpy as np
import matplotlib.pyplot as plt

Archivo Vsp en formato segy

segyfile = 'BaseCH172_OVSP_StackRaw_Data.segy'

f = segyio.open(segyfile)

Algunos atributos se pueden extraer del SEGY

coordenadas_x = f.attributes(segyio.TraceField.GroupX)[:]
coordenadas_y = f.attributes(segyio.TraceField.GroupY)[:]
coordenadas_z = f.attributes(segyio.TraceField.ReceiverGroupElevation)[:]

# Visualización del Text Header

f=segyio.open(segyfile,ignore_geometry=True)
print(segyio.tools.wrap(f.text[0]))

C 1 CLIENT: ECOPETROL S.A.    CONTRACTOR: SLB           CREW
C 2 CLIENT REP.:Helman Duque
C 3 WELL: CH172        Date:24-Dec-2022
C 4 WELL HEAD UTM COORD. X(m): 1044465.91m                  Y(m): 928071.19m
C 5 FIELD: PIAR
C 6 DRILL FLOOR (DF): 1589ft                           REFERENCE LEVEL : DF
C 7 SEISMIC DATUM: 1968 ft AMSL         REPLACEMENT VELOCITY:7874 ft/sec
C 8 SURVEY DESCRIPTION: OVSP    SOURCE: Vibroseis    DOWNHOLE RECEIVER: hDVS
C 9 SOURCE 1 OFFSET FROM WELLHEAD:2712.3 ft    AZIMUTH:276.95 deg
C10 SOURCE ELEVATION:1563 ft
C11 PROCESSING CONTRACTOR: SCHLUMBERGER      DESCRIPTION OF DATA: OVSP
C12 DATE PROCESSED: 2-Feb-2022
C13 RAW hDVS Stack
C14 PROCESSING FLOW:
C15 1. EDIT & STACK 2. PICK FIRST ARRIVAL
C16
C17
C18
C19
C20
C21 FIELD SUM       NAVIGATION SYSTEM               RECORDING PARTY
C22 Byte Locations:
C23 Field channel number (13-16) Shotpoint number (17-20)
C24 Ident number (21-24) Trace number (25-28)
C25 Reference elevation for receiver*100 (41-44)
C26 Reference elevation for source*100 (45-48)
C27 Measured depth of receiver below reference level*100 (53-56)
C28 True vertical depth of receiver below datum*100 (57-60)
C29 True vertical depth of receiver below reference level*100 (61-64)
C30 Picked first arrival time (msec) * 100 (65-68)
C31 X coordinate of source - referenced to wellhead*100 (73-76)
C32 Y coordinate of source - referenced to wellhead*100 (77-80)
C33 X coordinate of receiver - referenced to wellhead*100 (81-84)
C34 Y coordinate of receiver - referenced to wellhead*100 (85-88)
C35 Horizontal distance between source and detector (89-92)
C36 T gain exponent * 100 (97-100)
C37 Horizontal rotation angle*100(101-104) Vertical rotation angle*100
(105-108)C38 Length of the trace in samples (115-116)
C39 DATA AT RECORDED TIME
C40 END EBCDIC

Histograma de amplitudes de las trazas

plt.hist(np.array(f.trace).T)

(array([[  2.,  14., 180., ...,   9.,   0.,   0.],
        [ 12.,  52., 378., ...,  76.,   7.,   1.],
        [  2.,  14., 180., ...,   9.,   0.,   0.],
        ...,
        [ 12.,  52., 378., ...,  76.,   7.,   1.],
        [  2.,  14., 180., ...,   9.,   0.,   0.],
        [ 12.,  52., 378., ...,  76.,   7.,   1.]]),
 array([-901.47167969, -701.42370605, -501.37579346, -301.32781982,
        -101.27988434,   98.76806641,  298.81600952,  498.86395264,
         698.91192627,  898.95983887, 1099.0078125 ]),
 <a list of 308 BarContainer objects>)

png

Visualización registro VSP

with segyio.open(segyfile, "r") as f:
    # Carga las trazas en una matriz numpy
    trazas = np.array([f.trace[i] for i in range(f.tracecount)])

# Configuración para graficar las trazas sísmicas
clip = 5e+2
vmin, vmax = -clip, clip
plt.figure(figsize=(10, 6))
plt.imshow(trazas.T, aspect='auto', cmap='seismic', extent=[0, 308, 4000, 0], vmin=vmin, vmax=vmax)
plt.colorbar(label="Amplitud")
plt.xlabel("Número de traza")
plt.ylabel("Tiempo (ms)")
plt.title("Sección de Trazas Sísmicas")
plt.show()

png

Gráfico interactivo

import plotly.graph_objects as go

# Suponiendo que `trazas` es tu matriz con dimensiones (308, 4001)
# Crear el gráfico interactivo
clip = 1e+2
vmin, vmax = -clip, clip

fig = go.Figure(data=go.Heatmap(
    z=trazas.T,  # Transponemos para que las trazas sean verticales
    zmin=vmin,
    zmax=vmax,
    colorscale="RdBu",
    colorbar=dict(title="Amplitud"),
))

fig.update_layout(
    title="Sección de Trazas Sísmicas",
    xaxis_title="Número de traza",
    yaxis_title="Tiempo (ms)",
    yaxis=dict(autorange="reversed"),  # Invierte el eje Y para que el tiempo crezca hacia abajo
    height=600,
)

fig.show()
